NIXIE CLOCK - IV-11 & IV-22 VFD Clock

| Key_1 | Key_2 | Function |  |  |
| --- | --- | --- | --- | --- |
|  | CH- | Turn ON/OFF – tube Auto_Power_OFF mode (4 time intervals in day to OFF all tubes) |  |  |
|  | CH | Open „MENU“ (auto return after 10 sec.) | SET | Save actual configuration to EEPROM (valid after new RESET) |
|  |  |  | DEF | Set Default / Factory configuration from Flash to EEPROM (valid after new RESET) |
|  |  |  | Use , to select function and to confirm selected function ! |  |
|  | CH+ | MUTE ON/OFF – Beep and Alarm sound ON/OFF |  |  |
|  | ¦<< | In active EDIT mode select tubes position to change value. Selected tubes will flash. Repeated press button select: ALL tubes – Position 1 – Position 2 … - ALL tubes (auto return after 10 sec.) For selected tube position use , to change value and to confirm changes ! |  |  |
|  | >¦¦ | CLOCK | Switch ROLL (Time-Date-Temperature) view or only TIME view mode |  |
|  |  | TIME | 12 / 24 hour view mode (when all tubes flash) |  |
|  |  | DATE | Switch view order: YY_MM_DD / MM_DD_YY / DD_MM_YY (when all tubes flash) |  |
|  |  | TEMPERATURE | Switch between Celsius / Fahrenheit degrees mode (when all tubes flash) |  |
|  |  | Auto_Power_OFF | Select timer: b0, E0, b1, E1, b2, E2, b3, E3 (when all tubes flash) Timers: b0, b1, b2, b3 – Begin time (tubes OFF), E0, E1, E2, E3 – End time (tubes ON). Logical set: b0 <= E0 <= b1 <= E1 <= b2 <= E2 <= b3 <- E3. Other time settings doesn't make sense. |  |
|  | >>¦ | CLOCK mode | Adjust - increase brightness intensity, (auto return after 10 sec.) |  |
|  |  | EDIT mode | Set – increase value for TUBE in „blinking“ position, (auto return after 10 sec.) |  |
|  | 100+ | CLOCK mode | Adjust - decrease brightness intensity, (auto return after 10 sec.) |  |
|  |  | EDIT mode | Set - decrease value for TUBE in „blinking“ position, (auto return after 10 sec.) |  |
|  | - | The previous color of TUBE_LED in breathing mode (CLOCK mode, valid in next modes: Multi color BREATH – One color BREATH – one color ON) |  |  |
|  | + | STOP / Confirm ALARM (alarm sound OFF) Alarm sound sequence on ALARM match time: 1 min. SOUND, 1 min. SILENT, 1 min. SOUND, 1 min. SILENT, 1 min. SOUND, 1 min. SILENT, 2 min. SOUND, automatic OFF ALARM. |  |  |
|  | EQ | The next color of TUBE_LED in breathing mode (CLOCK mode, valid in next modes: Multi color BREATH – One color BREATH – one color ON) |  |  |
|  | 200+ | Confirm all changes in TIME, DATE, ALARM, ROLL, AUTO_POWER_OFF modes. System use only confirmed changes. You must use -> SET -> to write actual configuration to EEPROM. Only configuration from EEPROM is activated after system RESET ! |  |  |
|  | 0 | Switch next mode for DOT_LED ( Modes: LED OFF, LED Standby – Green) |  |  |
|  | 1 | Activate EDIT mode for „ TIME“ – all Tubes flash, (auto return after 10 sec.) Tube position: HH_MM_SS (HH – hours 12/24, MM – minutes, SS – seconds) |  |  |
|  | 2 | Activate EDIT mode for „DATE“ – all Tubes flash, (auto return after 10 sec.) |  |  |
|  | 3 | Activate EDIT mode for „ALARM“ – all Tubes flash, (auto return after 10 sec.) Tube position: A-_HH_MM / --_HH_MM (A- Alarm enabled, -- Alarm Disabled, HH – hours 12/24, MM – minutes) |  |  |
|  | 4 | Show „TEMPERATURE“ – all Tubes flash, (auto return after 10s) Tube position: _T_TT_tC / _T_TT_tF (C - Celsius, F – Fahrenheit, TTT – whole part, t - decimal part) |  |  |
|  | 5 | Activate tube „TEST“ mode – all Tubes flash, use or to change value [0,1..,F], (auto return after 10 sec.) Tube position: All tubes show same number, Number 0..7 flash tubes, 8..F don’t flash |  |  |
|  | 6 | Activate EDIT mode for „AUTO_POWER_OFF“ – all Tubes flash, (auto return after 10 sec.) Tube position: All tubes show same number, Number 0..7 flash tubes, 8..F don’t flash |  |  |
|  | 7 | Activate EDIT mode for „ROLL“ timers in „CLOCK“ mode – all Tubes flash, (auto return after 10 sec.) Left tubes: TIME (0-99 sec.), Middle tubes: DATE (0-99 sec.), Right tubes: TEMPERATURE (0..99 sec.) |  |  |
|  | 8 | Switch TUBE_LED mode: OFF – Multi color BREATH – One color BREATH – one color ON - Fire/Flame effect |  |  |
|  | 9 | Switch next mode for WEEK_LED ( Modes: OFF, Standby – Green, Temperature) |  |  |

21 key IR Remote Control

SAMPLES

24 Hour		  Minute		   	   Second

Year 			Month 			Date

(29.6C)

(86.5F)